Design Knowledge for Deep-Learning-Enabled Image-Based Decision Support Systems: Evidence From Power Line Maintenance Decision-Making

Julius Peter Landwehr, Niklas Kühl, Jannis Walk, Mario Gnädig

Business & Information Systems Engineering, 2022, Vol. 64, Issue 6, pp. 707–728. DOI: 10.1007/s12599-022-00745-z

Фрагмент для перевода: разделы 2.4–4 (до подраздела 4.1), с. 711–713. Надстрочные номера после терминов указывают номер термина в терминологическом словаре.

2.4 Image-Based Decision Support Systems1

The access to increasing volumes of images and the capabilities of DL2 to process and extract information from images creates the potential to harness this rich data and DL2 methods to facilitate effective decision-making (Chaudhuri and Bose 2020). Despite their capabilities, DL2 methods, particularly CNNs3, have found limited adoption in extant research of IS4 in general (Kraus et al. 2020), and specifically DSS5. Most research performed on image-based decision support focuses on the medical application domain6 (Ben-Cohen et al. 2017; Comaniciu et al. 1999). However, these works use highly specific medical scans rather than images from the visible spectrum. Some examples of the scarce literature on DL2-enabled image-based decision support in non-medical contexts include vision-based7 maintenance and monitoring applications or pattern analysis8 (Xie et al. 2020; Schumann et al. 2019; Chaudhuri and Bose 2020; Nazerdeylami et al. 2019; Jamshidi et al. 2018; Ren et al. 2020).

Despite the efficacy of DL2 methods for image processing in related decision support contexts, none of the previous work provides guidance on how to design IB-DSS1. Specifically, although all these studies aim for improved data and information availability, close to no insight is provided on how to bridge the gap between the sole image processing as well as consequent information extraction, and the respective efficient, high-quality decision-making.

2.5 Synthesis and Research Gap

This work aims to interweave two research domains. It combines the applied research of image processing in power line maintenance9 (PLM) with the need for decision support in vision-based7 domains in general and in PLM9 in particular. This allows us to tap new potential through making previously unattainable data and information from individual images available.

We address this potential by investigating the environment of automated vision-based7 PLC10 maintenance, focusing on the design of a holistic image-based decision support solution. We develop design knowledge for IB-DSS1 and evaluate it by instantiating a concrete artifact for PLC10 maintenance. We extend the reviewed existing works (cf. Table 1) by managing to detect11 PLCs10 of extreme size difference (insulators12 and safety pins13), which we believe is a crucial prerequisite for moving towards decision support in this domain.

3 Research Methodology

The research at hand develops design knowledge for IB-DSS1 which supports the maintenance decision-making and planning of maintenance engineers14 (MEs) for power lines15. Since design science research16 (DSR) has proven itself to be not only a suitable but also an important paradigm to develop IS4 in general (Gregor and Hevner 2013) and DSS5 in particular (Arnott and Pervan 2012), we follow its steps to develop and evaluate our artifact. At its core, DSR16 is a problem solving paradigm that involves two primary and distinct activities to design solutions to real-world problems: (1) the development of innovative artifacts in a series of design activities based on a deep understanding of the problem, justificatory knowledge, and the capabilities of the researcher and (2) the evaluation of the novel artifact to assess its ability and utility in solving the identified problem (Hevner et al. 2004). Following this “build-and-evaluate loop” (Hevner et al. 2004), we iteratively develop an artifact to extend the knowledge base17 regarding IB-DSS1.

Besides this loop – more precisely termed design cycle – Hevner (2007) describes the existence of two additional cycles: relevance and rigor. The three cycles are inherently related and part of any DSR16 project. The relevance cycle connects the environment, application domain6, or case company of the research project to the design science16 activities by, for instance, incorporating input from expert practitioners. It does not only provide the requirements, problems, or challenges for the research, but also defines acceptance criteria18 (Hevner and Chatterjee 2010). The rigor cycle relates the design science16 activities to the existing knowledge base17. It provides knowledge from scientific theories, engineering methods, experience, and expertise to the research project. The often repeatedly performed design cycle is the core of any DSR16 project and naturally builds on the insights from the two previous cycles. Specifically, during a design cycle the research iterates between construction and evaluation of an evolving artifact (Hevner and Chatterjee 2010) to eventually deploy19 the artifact in the environment as well as distill insights and output the research’s design knowledge contributions into the knowledge base17.

In the general view of our research displayed in Fig. 1 we start with studying the environment in which the research is embedded. We consequently state our application case (Sect. 4.1) and review related challenges and problems (Sect. 4.2). Joining these insights with knowledge from kernel theories we conceptualize principles and requirements for the problem class of IB-DSS1. We subsequently derive a concrete PLM9 artifact and, based on Turban et al. ’s (2010) high-level notion of a DSS5, first focus on the model component20 (MC) of our DSS5 artifact in the first design cycle (Sect. 5.2). Afterwards we move to the user interface component21 (UIC) in the second design cycle (Sect. 5.3). To orchestrate the evaluation of our artifact, we apply and follow the overarching Framework for evaluation in design science (Venable et al. 2016) to rigorously demonstrate the utility and efficiency of the artifact and its underlying design knowledge. Figure 1 provides an overview of the performed evaluation episodes (EE) in these design cycles. As it is our goal to indicate technical feasibility as well as utility of IB-DSS1 enabled through DL2, we start with a technical evaluation and then move to a naturalistic context within the application setting.

4 Application Case and Conceptualization

Our DSS5 artifact, built on images from the visible spectrum, intends to support MEs14 of power line15 infrastructure in their decision-making. More precisely, our system supports the planning and scoping of individual maintenance orders22 for the repair and replacement of components through improved data and information quality. Because the artifact is to intervene in an organizational context, it is considered “socio-technical23” (Gregor and Hevner 2013). To manage the complexity of the artifact construction in terms of size as well as social and technical components, Gregor and Hevner (2013) suggest the explicit extraction of design principles24 (DPs). We therefore conceptualize and suggest a number of tentative DPs24 for the design of artifacts of the problem class of IB-DSS1 by first investigating challenges in power line maintenance9 (PLM). These are recast into a prescriptive mode with appropriate abstraction yielding preliminary design requirements25 (DRs), which then serve as a basis for deriving the DPs24.